Module 3

Communication Satellites

Communication satellite acts as repeater stations that provide either point to point, point to multipoint or multipoint interactive services.

Communication related applications:
Telecommunication satellites provide a varied range of services mainly including

- Television broadcasting

- International telephony and

- Data communication services.

Satellite TV refers to the use of satellites for relaying TV programs from a point where they originate to a large geographical area. Geo satellites are employed for satellite TV applications via TVRO (Television Receive Only) and DBS (Direct Broadcast System).

Satellite telephony provides long distance point-to-point or trunk telephony services, as well as mobile telephony services.

Satellite also provides data communication services including data broadcast and multimedia services such as data collection and broadcasting, image and video transfer, voice, internet, two-way computer interactions and database inquiries.

Broadcasting services like TV, radio and telephony services remain in domain of Geo satellites & newer services like messaging, voice, fax, data & video conferencing suited to LEO, MEO or HEO satellite constellation.

Frequency bands:
Satellite communication employs EM waves for transmission of information between Earth and space. Various bands used in satellite communication include L, S, C, X, Ku, K & Ka.

Payloads:
Transponder is the key payload for any communication satellite.

Basic elements of satellite communication system are depicted below.

Figure 4.1 Basic elements of a satellite communication system

The information to be transmitted (voice channels for telephone service, a composite video signal or digital data) is modulated using analog or digital means, up-converted to its desired microwave frequency band, amplified to the required level and then beamed up to the satellite from the transmitting earth station.

The received signals are amplified by the satellite, down converted to a different frequency and then re-transmitted towards Earth. The device on board satellite performs amplification and frequency conversion known as transponder is the main payload for any communication satellite.

Satellites carry a number of these transponders varying from 10 to as many as 100 on high-capacity satellites.

Types of transponders:
Transponders may be broadly classified into the following types:

- Transparent or bent pipe transponders

- Regenerative transponders

Transparent or bent pipe transponders
This type of transponder provides only amplification, frequency translation and frequency re-translation. The modulation and spectral shape of the signals are not altered, as they simply transmit information back to earth. (Also referred as bent pipe transponders).

The full bandwidth is separated into channels by band-pass radio frequency filters.
The output of filters is amplified by separate power amplifiers (TWTA) and then combined and fed to common transmitting antennas for down-beaming the signals on the Earth.

Figure 4.2 Transparent transponders

Regenerative transponders:
Here some onboard processing is done and the received signal is altered before transmission.
The onboard processing helps to improve throughput and error performance.
The repeaters use various digital techniques like narrowband channel selection, routing, demodulation, error correction, separating of data for processing the received signal.

Satellite vs Terrestrial Network

The advantages of satellite over terrestrial network:

- Broadcast property – wide coverage area

- Wide bandwidth – high transmission speed & large transmission capacity

- Geographical flexibility – independence of location

- Easy installation of ground stations

- Uniform service characteristics

- Immunity to natural disasters

- Independence from terrestrial infrastructure

- Cost aspects – low cost per added station and distance-insensitive cost

Disadvantages w.r.t. terrestrial networks:

- Transmission delay

- Echo effects

- Launch & satellite cost

Satellite telephony:
Satellite provides both long distance point-to-point trunk telephony services as well as mobile telephony.
Potential uses of these services include people living in remote areas, international business travellers.
Satellite telephony networks employ point-to-point duplex satellite links enabling simultaneous communication in both directions.

Figure 4.3 Satellite point-to-point telephone networks (PBX, private branch exchange; PSTN, public switched telephone network)

Satellite TV

Satellite television is the most widely used and talked about application area of communication

satellites. In fact, it accounts for about 75%of the satellite market for communication services.

Satellite television basically refers to the use of satellites for relaying TV programmes from

a central broadcasting centre to a large geographical area. Satellites, by their very nature of

covering a large geographical area, are perfectly suited for TV broadcasting applications. As

an example, satellites like GE and Galaxy in the US, Astra and Hot Bird in Europe, INSAT

in India and JCSAT (Japanese communications satellite) and Superbird in Japan are used for

TV broadcasting applications. The five Hot Bird satellites provide 900 TV channels and 560

radio stations to 24 million users in Europe. Other means of television broadcasting include

terrestrial TV broadcasting and cable TV services. Satellites can provide TV transmission

services either directly to the users or in conjunction with the cable and terrestrial broadcasting

networks. This will be explained in detail in the paragraphs to follow.

4.1.1 A Typical Satellite TV Network

Satellite television employs GEO satellites acting as point-to-multipoint repeaters receiving

a certain telecast from the transmission broadcasting centre and retransmitting the same after

frequency translation to the cable TV operators, home dishes, etc., lying within the footprint of

the satellite. Satellites can provide TV programmes either directly to the users (direct-to-home

television) or indirectly with the help of cable networks or terrestrial broadcasting networks,

where the satellite feeds the signal to a central operator who in turn transmits the programmes

to the users either using cable networks or through terrestrial broadcasting. A typical satellite

TV network, like any other satellite network, can be divided into two sections: the uplink

section and the downlink section.

Figure 4.4 Uplink section of satellite TV networks

The uplink section (Figure 4.4) comprises three main components: the programming source,

the broadcasting centre and the main broadcasting satellite. The programming source comprises

TV channel networks, cable TV programmers, etc., that provide various TV programming

signals, like TV channels, sports coverage, news coverage or local recorded TV programmes, tothe broadcasting centre either through terrestrial means, like using the line-of-sight microwave

communication and the fibre optic cable, or using satellites referred to as back-haul satellites.

As an example, for one-time events like various news events, a vehicle-mounted Earth station

generally operating in theKu band is driven to the site and then the programmes are transmitted

to the main broadcast centre using a back-haul satellite on a point-to-point connectivity basis

[known as satellite news gathering (SNG)]. In the case of a live telecast of certain events like

sports, the signals picked up by the cameras are transmitted to the main broadcasting centre

either with the help of a microwave link, a fibre optic link or a point-to-point backhaul satellite

link. The broadcasting centre is the hub of the satellite TV system and it processes and beams

the signal to the main broadcasting satellite. It also adds commentary or advertisements to the

signals from the various programming sources. Generally, the signals are transmitted using

analogue techniques in the C band or using a digital format employing various compression

techniques in the Ku band. The signals are also generally encrypted before transmission to

prevent unauthorized viewing.

The satellite downlink comprises the main broadcasting satellite and the TV receiving network.

In fact, the main broadcasting satellite is common in both the uplink and the downlink

sections. The receiver network in the case of satellites distributing programmes to the terrestrial

broadcast network comprises various terrestrial broadcasting centres that receive the satellite

signal and transmit them to the users in the VHF and the UHF bands using terrestrial broadcasting.

The user end has directional yagi antennas to pick up these signals. In the case of satellite

distributing programmes to a cable operator, the downlink section comprises the cable–TV

head ends and the cable distribution network. For DTH services, receive-only satellite dishes

are mounted at the user’s premises to receive the TV programmes directly from the satellite.

4.1.2 Satellite–Cable Television

As mentioned earlier, cable TV refers to the use of coaxial and fibre optic cables to connect

each house through a point-to-multipoint distribution network to the head end distribution

station. Cable TV, originally referred to as CATV (community antenna television) stood for

a single head end serving a particular community, like various houses in a large building.

The present-day cable TV system is more complex and involves a larger distribution area.

The head ends receive programming channels from either a local broadcasting link or through

satellites. The use of satellites to carry the programming channels to the cable systems head

ends is referred to as satellite–cable television (Figure 4.5). The head end in this case consists

of various receive-only Earth stations with the capability of receiving telecast from two to

six satellites. These Earth stations either have multiple receiving antennas or, a single dish

antenna with multiple feeds, with each feed so aligned as to receive telecast from a different

satellite.

The transmission from the satellite is either in the analogue format (mainly in the C band)

or in the digital format (mainly in the Ku band). In analogue format of transmission, each

receiver is tuned to a different transponder channel and the signals from various receivers are

Figure 4.5 Satellite cable television

multiplexed for transmission to the users. The channels received in the digital format can be

transmitted either digitally or in the analogue form as mentioned above. This processed digital

or analogue information is then transmitted over a typical cable distribution network to a large

number of houses known as subscribers, who pay a monthly fee for the service. The cable

operators scramble their programmes to prevent unauthorized viewing. The receiving end then

consists of a set top box to descramble and retrieve the original signal. The cable TV operators

also transmit the videotaped recorded programmes from other sources in addition to showing

programmes received from the satellites.

4.1.3 Satellite–Local Broadcast TV Network

It is the same as the satellite–cable TV network except for the fact that here the satellite

distributes programming to local terrestrial broadcasting stations instead of distributing it to

the cable head end stations. The broadcasting stations use powerful antennas to transmit the

received signals to various users within the line-of-sight (50–150 km) using UHF and VHF

microwave bands. The users receive these TV signals using directional antennas like yagi

antennas, reflector antennas or dipole antennas. A typical satellite–local broadcast network

is shown in Figure 4.6. Sometimes, a combination of both the satellite-cable TV and the

satellite-local broadcast TV networks is used for distributing TV programmes to the users. As

an example, one of the possible configurations is where the satellite sends the signals to the

Figure 4.6 Typical satellite local-broadcast TV network

local broadcasting stations, which in turn broadcast them to the cable operators.

4.1.4 Direct-to-Home Satellite Television

Direct-to-home (DTH) satellite television refers to the direct reception of satellite TV programmes

by the end users from the satellite through their own receiving antennas (Figure 4.7).

Figure 4.7 Direct-to-home satellite television

DTH services can be broadly classified into two types, namely the television receive-only

(TVRO) and the direct broadcasting satellite (DBS) services, depending upon the frequency

band utilized and the size of the receiving antennas. TVRO systems operate in the C band

whereas the DBS systems operate in the Ku band. In the present context, when DTH systems

are mentioned, more often than not it refers to the DBS systems only.

4.1.5. Television Receive-Only (TVRO) Services

TVRO systems employ large dishes (6 to 18 feet across) placed in the user’s premises for

the reception of analogue signals from the satellite operating in the C band. The antenna size

is larger in this case as compared to DBS systems as the wavelength at C band frequencies

is larger than at the Ku band frequencies. In addition, international and domestic regulations

limit C band power because of the possibilities of RF interference between the satellite and

the microwave links operating in the C band. Generally, each C band transponder provides one

analogue TV channel, and hence a satellite with 16 such transponders will be able to support

only 16 TV channels. Hence, for complete channel coverage, the TVRO receiver antenna must

have a steerable dish. These systems are made user friendly by using microprocessor control,

allowing the viewer to select the desired channel with a remote-control unit. The antenna

then moves automatically using electronic control methods to point to the desired satellite.

TVROs are based on open standard equipment and provide the largest variety of TV programs,

including cable TV programs, foreign stations, free programming channels and live unedited

feeds between broadcasting stations like news, sports, etc.

A look into the development of DTH services suggests that TVROs were the original form of

DTH satellite TV reception. The concept of using dishes at the user’s premises to view satellite

television directly started in early 1980s, when people in the USA started putting dishes in

their backyards for direct satellite reception. TVROs reached their peak around the year 1994

and have slowly given way to direct broadcasting satellite (DBS) services. However, TVRO

systems still exist and are being updated to receive digitally scrambled programming channels

from Ku band satellites.

Receive only home TV systems: DBS TV/FM reception

This consists of a receiving antenna feeding directly into LNA unit (normally 950–1450 MHz).
The signal is fed into a tracking filter, which selects desired channel.
The selected channel is again down-converted from 950–1450 MHz to fixed IF of 70 MHz, which is demodulated and baseband information is used to generate a VSB signal which is fed into one of VHF/UHF channels of standard TV set.

DBS: Direct Broadcast System

DBS includes audio, television, and internet services. DBS TV is also called digital TV.

The home receiver consists of two units: an outdoor unit and an indoor unit.

The down link signal is received by the antenna into LNB receive horn.

Downconverter converts 12.2 – 12.7 GHz band to 950 – 1450 MHz frequency band, shifted to transmission through connecting cable to RF in.

The carrier is demodulated, QPSK modulation is being converted to a bit stream. Error correction is carried out in the decoder block. The demultiplexer separates individual programs, which is stored in buffer memories for further processing, such as conditional access.

A typical satellite TV network

Diagram shown with: Back haul satellite, Broadcast Center, Main TV broadcasting satellite, Terrestrial broadcast, Sports event or SIT or a TV channel provider.

Uplink section consists of 3 main components:

- programming source

- the broadcast center

- main broadcasting satellite

Programming source comprises TV channel networks, cable TV programmers, etc. that provide various TV programming signals like TV channels, sports coverage, news coverage, or local recorded TV programs to broadcast center either through terrestrial means like line-of-sight microwave link, fiber optic cable or using satellite referred to as back-haul satellite.

Satellite Radio
A satellite providing high-fidelity audio broadcast service to the broadcast radio stations is referred to as satellite radio. Sound quality is excellent in this case due to wide audio bandwidth (15 kHz) and low noise provided over the satellite link.

Satellite Data Communication Services
Data communication services refer to unidirectional or bidirectional data broadcasting services provided by GEO satellites through VSAT networks.

Very Small Aperture Terminals (VSAT)
VSATs are used to provide one-way or two-way data broadcasting services, point-to-point voice services, and one-way video broadcasting services.
VSATs are ideal for centralized networks.

Typical examples are:

- small and medium business with central office

- banking institutions with branches all over the country

- reservation system & airline ticketing system

Typical VSAT network ground segment consists of high-performance hub earth stations and a large number of low-performance terminals referred to as VSATs. The space segment

comprises a GEO satellite link between hub stations & VSAT terminals.

Figure 4.8 A typical VSAT network

The hub station is normally large, high-performance earth station consisting of outdoor antenna (6–10 m diameter), RF terminal to provide uplink, backend equipment like modems, multiplexers, encoders, a control center to monitor and manage networks.

VSAT terminals are smaller, comprising of outdoor antenna (0.5 m to 2.4 m diameter), RF equipment (LNB and baseband equipment). VSAT networks employ C-band & Ku-band for transmission & reception.

VSAT networks generally employ TDMA scheme for data transmission.

VSAT network topologies

Regional Satellite Systems

One of the drawbacks of international satellite systems is that they are not optimized to the needs of the individual countries. Regional satellite missions were established with an aim of strengthening the communication resources of countries belonging to the same geographical area.

Examples:

- Eutelsat

- Arabsat

- Asiasat

- Measat

- Asia Cellular Satellite

- Thuraya

Eutelsat (European Telecommunication Satellite Organization)

This provides television, telephone and data transmission services on regional basis. 20 satellites in the GEO orbit operated by Eutelsat covering 98% of world population.

Eutelsat-1F series, Eutelsat-2F series and Eutelsat-W series have been launched. Eutelsat covers Europe, Asia and Africa.

National Satellite System:

This service refers to a domestic satellite system, provide services to a particular country. National satellite systems were originally developed/established by the United States, USSR and Canada, depending upon specific needs. Later developing countries like India, China, Japan etc. have their own satellite systems.

Examples:

- Galaxy, Satcom, Echostar, Telestar of U.S.

- Brasilsat of Brazil

- INSAT of India

- Optus of Australia

- Sinosat of China

INSAT (Indian National Satellite) is joint venture of Department of Space (DOS) of India, Dept. of Telecom of India (DOT), Indian Meteorological Dept. (IMD), All India Radio and Doordarshan (AIR).

Various series are:

- INSAT-1A, 1B, 1C, 1D

- INSAT-2A, 2B, 2C, 2D, 2E

- INSAT-3B, 3C, 3A, 3E

- INSAT-4A, 4B, 4CR